(2nd program)

int buttonPin = 2;

int ledPin = 13;

bool buttonState = false;

void setup() {

pinMode(ledPin, OUTPUT);

pinMode(buttonPin, INPUT);}

void loop() {

buttonState = digitalRead(buttonPin);

if (buttonState == HIGH) {

digitalWrite(ledPin, HIGH);

} else {

digitalWrite(ledPin, LOW); }}

(3rd program)

const int ldrPin = A0;

const int ledPin = 9;

int ldrValue = 0;

int ledBrightness = 0;

void setup() {

Serial.begin(9600);

pinMode(ledPin, OUTPUT);}

void loop() {

ldrValue = analogRead(ldrPin);

ledBrightness = map(ldrValue, 0, 1023, 255, 0);

ledBrightness = constrain(ledBrightness, 0, 255);

analogWrite(ledPin, ledBrightness);

Serial.print("LDR Value: ");

Serial.print(ldrValue);

Serial.print(" | LED Brightness: ");

Serial.println(ledBrightness);

delay(200);

}

(4th program)

const int sensorPin = A0;

int sensorValue = 0;

float voltage = 0.0;

float temperatureC = 0.0;

void setup() {

Serial.begin(9600);}

void loop() {

sensorValue = analogRead(sensorPin);

voltage = sensorValue * (5.0 / 1023.0);

temperatureC = voltage * 100.0;

Serial.print("Temperature: ");

Serial.print(temperatureC);

Serial.println(" °C");

delay(1000);}

(5th program)

const int relayPin = 8;

int myCount = 1;

float myTemp = 101.0;

void setup() {

Serial.begin(9600);

pinMode(relayPin, OUTPUT);

digitalWrite(relayPin, LOW);

Serial.println("System Initialized");

Serial.println("RELAY OFF");}

void loop() {

Serial.print(myCount);

Serial.print(" Temp: ");

Serial.print(myTemp, 1);

Serial.println(" F");

if (myTemp > 100.0) {

digitalWrite(relayPin, HIGH);

Serial.println("RELAY ON");

delay(2000);

digitalWrite(relayPin, LOW);

Serial.println("RELAY OFF"); }

myCount++;

delay(5000);

}

(6th program)

void setup() {

Serial.begin(9600);

Serial.println("System Initialized");

Serial.println("Connecting to WiFi...");

delay(1000);

Serial.println("WiFi Connected!");

Serial.println("IP Address: 192.168.1.100");}

void loop() {

Serial.println();

Serial.println("Sending HTTPS GET request...");

Serial.println("URL: https://api.quotable.io/random");

delay(500);

Serial.println("HTTP Response Code: 200");

Serial.println("JSON data received.");

Serial.println("JSON parsed successfully.");

Serial.println();

Serial.println("----- Retrieved Information -----");

Serial.println("Quote: The future depends on what you do today.");

Serial.println("Author: Mahatma Gandhi");

Serial.println();

Serial.println("----- NTP Time -----");

Serial.println("Date: Tuesday");

Serial.println("Time: 10:30:00");

delay(5000);}

(7th program)

const int sensorPin = A0;

int sensorValue = 0;

float voltage = 0.0;

float temperatureC = 0.0;

void setup() {

Serial.begin(9600);

Serial.println("Environmental Monitoring System");

Serial.println("Temperature Sensor Initialized");

Serial.println("--------------------------------");

}

void loop() {

sensorValue = analogRead(sensorPin);

voltage = sensorValue * (5.0 / 1023.0);

temperatureC = (voltage - 0.5) * 100.0;

Serial.print("Temperature: ");

Serial.print(temperatureC, 1);

Serial.println(" C");

Serial.println("--------------------------------");

delay(2000);

}

(8th program)

const int soilSensorPin = A0;

int soilValue = 0;

String moistureStatus = "";

void setup() {

Serial.begin(9600);

Serial.println("Soil Moisture Monitoring System");

Serial.println("--------------------------------");

}

void loop() {

soilValue = analogRead(soilSensorPin);

if (soilValue < 400) {

moistureStatus = "WET";

} else if (soilValue > 700) {

moistureStatus = "DRY";

} else {

moistureStatus = "OK";

}

Serial.print("Raw Sensor Value: ");

Serial.println(soilValue);

Serial.print("Soil Status: ");

Serial.println(moistureStatus);

Serial.println("--------------------------------");

delay(1000);

}